SALP / ESA Sea Level CCI

Validation Report:WP2500 Regional SSH bias corrections between altimetry missions

| Reference: | CLS-DOS-NT-11-227 |
| --- | --- |
| Nomenclature: | SALP-NT-MA-EA-22005-CLS |
| Issue: | 1. 1 |
| Date: | Mar. 12, 12 |

| Chronology Issues: |  |  |  |
| --- | --- | --- | --- |
| Issue: | Date: | Reason for change: | Author |
| 1.0 | 29/09/11 | Document creation | M. Ablain (CLS) |
| 1.1 | 12/03/12 | Minor corrections | M. Ablain (CLS) |

| People involved in this issue: |  |  |
| --- | --- | --- |
| Written by (*): | M. Ablain (CLS) | Date + Initials:( visa or ref) |
| Checked by (*): | S. d’Alessio | Date + Initials:( visa or ref) |
| Approved by (*): |  | Date + Initials:( visa or ref) |
| Application authorized by (*): | CNES | Date + Initials:( visa or ref) |

*In the opposite box: Last and First name of the person + company if different from CLS

| Index Sheet: |  |
| --- | --- |
| Context: | Baghera tool, project ACT-OCEAN |
| Keywords: | Oceanography, sea level |
| Hyperlink: |  |

| Distribution: |  |  |
| --- | --- | --- |
| Company | Means of distribution | Names |
| CLS | Notification | DOS-MSA |
| CNES | Electronic file | E. BRONNER T. GUINLE A. GUILLOT J.-L. MESTRE |

List of tables and figures

List of tables:

Aucune entrée de table d'illustration n'a été trouvée.

List of figures:

Figure 1 : Map of SLA differences centered on the average between Jason-1 and Jason-2 during the Jason-2 verification phase	2

Figure 2 : SLA differences between Jason-1 and Jason-2 versus the latitudes from the measurements (red curve) and after adjusting a polynomial function (blue curve)	3

Figure 3 : Map of SLA differences between Jason-1 and Jason-2 without applying the regional correction (just centered on the average) on top and after applying it on bottom.	4

Figure 4 : Map of SLA differences centered on the average between TOPEX-B and Jason-1 during the Jason-1 verification phase applying all corrections (on top) and only SSB correction (on bottom).	7

Figure 5 : Map of SLA differences centered on the average between TOPEX-B and Jason-1 during the Jason-1 verification phase for descending passes (on top) and for ascending passes (on bottom).	8

Figure 6 : SLA differences between TOPEX-B and Jason-1 versus the latitudes selecting all the measurements (red curve), the ascending passes (green curve) and the descending passes (blue curve).	9

Figure 7 : SLA differences between TOPEX and Jason-1 (in red) with polynomial functions superimposed versus the latitudes for ascending passes.	10

Figure 8 : SLA differences between TOPEX and Jason-1 (in red) with polynomial functions superimposed versus the latitudes for descending passes	10

Figure 9 : Map of SLA differences between TOPEX and Jason-1 for ascending passes without applying the regional correction (just centered on the average) on top and after applying it on bottom.	12

Figure 10 : Map of SLA differences between TOPEX and Jason-1 for descending passes without applying the regional correction (just centered on the average) on top and after applying it on bottom.	13

Figure 11 : Map of SLA differences between TOPEX and Jason-1 without applying the regional correction (just centered on the average) on top and after applying it on bottom.	14

Figure 12 : Map of SLA differences between TOPEX-A and TOPEX-B for ascending (on bottom) and descending (on top) passes (TOPEX-A and TOPEX-B periods respectively cover cycles 231 to 235 and 236 to 240).	16

Figure 13 : Map of SSH differences between TOPEX-A and TOPEX-B for ascending (on bottom) and descending (on top) passes derived from TOPEX/ERS-2 crossovers differences (TOPEX-A and TOPEX-B periods respectively cover cycles 231 to 235 and 236 to 240).	17

Figure 14 : Map of SSH differences by band of latitudes between TOPEX-A and TOPEX-B for ascending (on bottom) and descending (on top) passes derived from TOPEX/ERS-2 crossovers differences (TOPEX-A and TOPEX-B periods respectively cover cycles 231 to 235 and 236 to 240)	18

Figure 15 : SLA differences between TOPEX-B and Jason-1 versus the latitudes selecting all the measurements (red curve), ascending passes (green curve) and descending passes (blue curve).	19

Figure 16 : MSL trend differences from 1993 to 2010 between MSL time series without and with regional bias corrections on TOPEX/Jason-1 and Jason-1/Jason-2	20

Figure 17 : MSL trend differences for ascending passes from 1993 to 2010 between MSL time series without and with regional bias corrections on TOPEX/Jason-1 and Jason-1/Jason-2	21

Figure 18 : MSL trend differences for descending passes from 1993 to 2010 between MSL time series without and with regional bias corrections on TOPEX/Jason-1 and Jason-1/Jason-2	21

Figure 19 : Global MSL trend differences from 1993 to 2010 between MSL time series without and with regional bias corrections on TOPEX/Jason-1 and Jason-1/Jason-2	22

List of items to be confirmed or to be defined

Lists of TBC:

Aucune entrée de table des matières n'a été trouvée.

Lists of TBD:

Aucune entrée de table des matières n'a été trouvée.

Applicable documents

AD 1 Sea level CCI project Management Plan
CLS-DOS-NT-10-013

Reference documents

RD 1 Manuel du processus Documentation
CLS-DOC

List of Contents

1. Introduction	1

2. Estimation of global bias and impact on MSL trend	1

3. Estimation of the regional MSL bias between Jason-1 and Jason-2	2

3.1. Description of the regional MSL bias	2

3.2. Specification of the correction	3

3.3. Assessment of the correction	4

4. Estimation of the regional MSL bias between TOPEX-B and Jason-1	6

4.1. Description of the regional MSL bias	6

4.2. Specification of the correction	8

5. Assessment of the correction	11

6. Estimation of the regional MSL bias between TOPEX-A and TOPEX-B	15

6.1. Description of the regional MSL bias	15

6.2. Specification of the correction	19

7. Impact of regional bias corrections on the whole MSL time series	20

8. Conclusions	23

Appendix A - Definition of the indicator value	24

Appendix B - Definition of altimetry standards	25

# Introduction

A main source of error to calculate the global or the regional Mean Sea Level (MSL) trends is the uncertainty to link together all the altimetry missions in order to provide a continuous MSL time series from 1993 onwards.

A very strong effort has been already done to link together as well as possible the global MSL altimetry missions deduced from TOPEX/Poseidon (T/P), Jason-1 and Jason-2 (this work is presented in this report).  At the moment, the uncertainty due to this global bias is close to 0.2 mm/yr which is not negligible for MSL studies.

Concerning the regional MSL bias at basin scales, systematic geographical biases between altimetry missions have been already detected and not taken into account (except between Jason-1 and Jason-2) to calculate sea-level maps over all the altimetry periods. These errors could impact directly the regional estimation of the MSL trends with a potential effect locally higher than 1 mm/yr, significantly stronger than for the global MSL (0.2 mm/yr).

Therefore, the objective of this study is to provide new corrections containing the regional MSL biases between NASA/CNES missions (T/P, Jason-1, Jason-2). In this report we describe how the corrections are calculated and their impact on regional MSL trends.

# Estimation of global bias and impact on MSL trend

At the moment the MSL time series is referenced to the three NASA/CNES missions: TOPEX/Poseidon (T/P), Jason-1, Jason-2.  But the T/P mission has to be separate into 2 phases TOPEX-A and TOPEX-B corresponding to the change of the altimeter instrument. Therefore, the MSL time series is composed on 4 time series that have to be linked together:  TOPEX A and TOPEX B (February 1999), TOPEX-B and Jason-1 (April 2003), Jason-1 and Jason-2 (October 2008).

The global biases between these missions have been accurately calculated (Ablain et al, 2009). It strongly depends on the altimetry standards used to calculate the SSH which have to be clearly defined (see table in annex). Between TOPEX-B and Jason-1 and between Jason-1 and Jason-2, global biases can be accurately calculated thanks to the Jason-1 and Jason-2 verification phases (at the beginning of each mission) where both satellites were respectively on the same ground track than T/P and Jason-1 ones, and spaced out by 72 seconds and 54 seconds. The global biases and errors deduced are for:

- TOPEX-B/Jason-1 (April 2003) : 84.5 mm ± 1 mm

- Jason-1/Jason-2 (October 2008) : 74.6 mm  ± 0.5 mm

The error between Jason-1 and Jason-2 (0.5 mm) is lower than between TOPEX-B and Jason-1 (1 mm) because the ground processing between Jason-1 and Jason-2 is more homogeneous than with TOPEX-B as the orbit calculation for instance, but especially the instrumental parameters (range, SWH, Sigma-0,…).

For TOPEX-A and TOPEX-B, they are no overlapping between both datasets to accurately estimate the bias. Furthermore, a strong decrease of the MSL evolution during this period is observed in relationship with “La Ninã” 1999 preventing an accurate SSH bias calculation between TOPEX-A and TOPEX-B. Therefore, the global bias has been estimate with an uncertainty stronger:

- TOPEX-A/TOPEX-B (April 2003) : 11.7 mm  ± 2 mm

All, the uncertainty associated to each bias is large enough to significantly affect the global MSL trend. Considering extreme bias errors, the global MSL trend is ranging from 2.8 to 3.3 mm/yr (Ablain et al, 2009). Therefore, a realistic error of ±0.2 mm/yr on the global MSL trend could be deduced. Notice that the impact of the SSH bias uncertainty is depending on the period and thus reduced with a longer period.

Summary: The global MSL bias to link the altimetry missions together are : - TOPEX-A/TOPEX-B (April 2003) : 11.7 mm ±2 mm - TOPEX-B/Jason-1 (April 2003) : 84.5 mm ±1 mm - Jason-1/Jason-2 (October 2008) : 74.6 mm ±0.5 mm The impact of the SSH bias uncertainties is ±0.2 mm/yr on the global MSL trend

# Estimation of the regional MSL bias between Jason-1 and Jason-2

## Description of the regional MSL bias

The correction of the regional MSL bias between Jason-1 and Jason-2 have been already calculated and applied in the last AVISO release concerning the sea-level map dataset for delayed-time. The description of this correction is presented here.

The map (Figure 1) of Sea Level Anomaly (SLA) differences between both missions is calculated during the Jason-1 verification phase (from Jason-2 cycles 1 to 20). It mainly highlights hemispheric differences (<1 cm) between both missions due to differences in orbit calculation (Ablain, 2010, Marine Geodesy).

Figure 1 : Map of SLA differences centered on the average between Jason-1 and Jason-2 during the Jason-2 verification phase

## Specification of the correction

It’s important to mention that although these hemispheric differences are small (<1 cm), they significantly impact the regional MSL trends (±0.5 mm/yr) as long as they are not corrected. Therefore, the objective is to produce a correction taking this regional bias error into account as well as possible. As the map of SLA differences between both missions is slightly noised and the ocean is not totally covered over the period of the Jason-2 verification phase (due to the ice coverage variation), it’s not possible to apply directly this map as a regional bias correction. Therefore, it would be better to modelize the hemispheric signal adjusting a polynomial function.

The SLA differences are first averaged by band of latitudes and plotted (Figure 2). A polynomial function (degree 4) is then fitted on the SLA differences and we obtained these coefficients (unit is cm):

Finally, this polynomial function corresponds to the correction which will be applied on the Jason-2 SSH bias to remove the global and the regional biases between Jason-1 and Jason62:

Figure 2 : SLA differences between Jason-1 and Jason-2 versus the latitudes from the measurements (red curve) and after adjusting a polynomial function (blue curve)

## Assessment of the correction

In order to assess the correction, a grid containing the value of the polynomial function is calculated and applied to the Jason-1/Jason-2 SLA differences. The impact is plotted in Figure 3. The average differences are small (<0.5 cm) and mainly due to the orbit error. There are no systematic differences.

The global bias of the regional correction has also to be consistent with the global bias defined at the beginning of the document. It is calculated taking into account the cell surface of the grid containing the correction. We obtained a value of 75.2 mm whereas the global bias was estimated to 74.6 mm. The 0.6 mm of differences are due to the period which is different to calculate the global and the regional biases: only 11 cycles are used centered on Jason-2 cycle number 11 to calculate the global bias. Therefore, in order to be consistent with the global bias, the constant of the polynomial function has to be reduced by 0.2 mm: it becomes 7.68 cm instead of 7.74 cm.

Figure 3 : Map of SLA differences between Jason-1 and Jason-2 without applying the regional correction (just centered on the average) on top and after applying it on bottom.

Conclusion: The SSH bias correction to adjust the Jason-2 SSH on the Jason-1 SSH is : With: Notice that this empirical correction strongly depends on the altimetry standards used on the Jason-1 or Jason-2 MSL calculation. If one of them is modified, especially concerning the orbit calculation, the coefficient of the polynomial function has to be revisited.

# Estimation of the regional MSL bias between TOPEX-B and Jason-1

## Description of the regional MSL bias

At the moment, only a global MSL bias is applied to link the TOPEX-B and Jason-1 MSL time data series in AVISO products. As previously described this value has been estimated to 84.5 mm ±1 mm. But regional differences have been also detected between these both missions (Ablain et al, OSTST Hobart 2007; Labroue et al, OSTST Seattle 2009) and they could be corrected.

As for Jason-1 and Jason-2, during the Jason-1 validation phase (from January to August 2002), TOPEX-B and Jason-1 SSH are in theory exactly the same (both missions are spaced out by 72 seconds on the same ground track). Therefore, they can be compared over this period in order to estimate the geographical bias.

The map of SLA differences, displayed on Figure 4 (top), highlights small differences between ±1 cm. They are systematic differences due the wet troposphere correction (derived from radiometers) in coastal areas. In order not to take into account theses inconsistencies between both missions, we directly calculated the SLA differences without applying the corrections in the SSH calculation except the SSB: Orbit – Range – SSB –MSS. Then we obtained the bottom map on Figure 4 where differences are reduced allowing a better observation of the residual differences.

The separation of these maps between ascending and descending passes clearly shows a behavior completely different and almost opposed (Figure 5). A strong hemispheric signal (± 2 cm) is observed on ascending passes whereas the SLA differences are more homogenous on descending passes. We also observed a different value of the bias in a narrow band at the equator.

The source of these strong regional biases (ascending/descending and North/South: differences by “quadrant”) is mainly due to ground processing differences between both missions as a result of a “leakage” anomaly in TOPEX waveforms (for more details, please refer to Labroue et al, OSTST Seattle 2009). Other lower discrepancies are also explained by differences between orbit calculation and SSB corrections.

Figure 4 : Map of SLA differences centered on the average between TOPEX-B and Jason-1 during the Jason-1 verification phase applying all corrections (on top) and only SSB correction (on bottom).

Figure 5 : Map of SLA differences centered on the average between TOPEX-B and Jason-1 during the Jason-1 verification phase for descending passes (on top) and for ascending passes (on bottom).

## Specification of the correction

In order to correct the hemispheric bias observed between both missions, the SLA differences (without correction except the SSB) are calculated by band of latitudes (Figure 6), selecting all the passes and separating ascending and descending passes. As previously mentioned, the behavior of ascending and descending passes is opposed in each hemisphere with a similar feature close to the equator.

Figure 6 : SLA differences between TOPEX-B and Jason-1 versus the latitudes selecting all the measurements (red curve), the ascending passes (green curve) and the descending passes (blue curve).

It seems judicious to propose a regional bias correction separating the ascending and descending passes, in order to also take it into account in along-track products. For map products, the interest is reduced since ascending and descending passes are roughly averaged inside a box.

Because of the discontinuity at the equator and these ascending/descending effects, the correction is defined by type of passes, for the north hemisphere, the south hemisphere and the equator band.

For ascending passes the polynomial correction is therefore the following:

And for descending passes, it is:

These polynomial functions correspond to the correction which will be applied on the TOPEX SSH to remove the global and regional biases between TOPEX and Jason-1:

The polynomial corrections are plotted on the both following figures for ascending and descending passes:

Figure 7 : SLA differences between TOPEX and Jason-1 (in red) with polynomial functions superimposed versus the latitudes for ascending passes.

Figure 8 : SLA differences between TOPEX and Jason-1 (in red) with polynomial functions superimposed versus the latitudes for descending passes

# Assessment of the correction

In order to assess the correction, a grid containing the value of the polynomial function is calculated and applied to the TOPEX/Jason-1 SLA differences. The impact is plotted in Figure 9 and Figure 10 for respectively ascending and descending passes (without applying any correction). The average differences are small (<0.5 cm) and mainly due to the orbit error. They are not systematic.

As previously, the global bias of the regional correction has also to be consistent with the global bias defined at the beginning of the document. It is calculated taking into account the cell surface of the grid containing the correction. We obtained a value of 91.3 mm whereas the global bias has been estimated to 84.5 mm. The 6.8 mm of differences are due to the period which is different to calculate the global and the regional biases: only 11 cycles are used centered on Jason-1 cycle number 11 to calculate the global bias. Therefore, in order to be consistent with the global bias, the constant of the polynomial function has to be reduced by 6.8 mm on ascending and descending passes.

Figure 9 : Map of SLA differences between TOPEX and Jason-1 for ascending passes without applying the regional correction (just centered on the average) on top and after applying it on bottom.

Figure 10 : Map of SLA differences between TOPEX and Jason-1 for descending passes without applying the regional correction (just centered on the average) on top and after applying it on bottom.

Figure 11 : Map of SLA differences between TOPEX and Jason-1 without applying the regional correction (just centered on the average) on top and after applying it on bottom.

Conclusion: The SSH bias correction to adjust the TOPEX SSH on the Jason-1 SSH is : With for ascending passes : And for descending passes: Notice that this empirical correction strongly depends on the altimetry standards used on the Jason-1 or TOPEX MSL calculation. If one of them is modified, especially concerning the orbit calculation, the coefficient of the polynomial function should be revisited.

# Estimation of the regional MSL bias between TOPEX-A and TOPEX-B

## Description of the regional MSL bias

At the moment, only a global MSL bias is applied to link TOPEX-A and TOPEX-B MSL time data series in AVISO products. As previously described this value has been estimated to 11.7 mm ±2 mm using the altimetry standards defined in TOPEX AVISO products. The value of this global bias is very sensitive to the sea state bias correction applied on each altimeter side.

Contrary to Jason-1/TOPEX and Jason-2/Jason-1, there are no over-lapping phase between TOPEX-A and TOPEX-B. Therefore, the map of SLA differences cannot be calculated over the same period but using a 50-day period before and after the TOPEX altimeter change on 10th of February 1999. But the maps of SLA differences obtained highlight strong areas of high oceanic variability (see Figure 12): they cannot provide reliable information on the regional MSL bias between TOPEX-A and TOPEX-B which is lower than the average oceanic variability between both periods.

Figure 12 : Map of SLA differences between TOPEX-A and TOPEX-B for ascending (on bottom) and descending (on top) passes (TOPEX-A and TOPEX-B periods respectively cover cycles 231 to 235 and 236 to 240).

The method proposed to highlight a potential regional bias between TOPEX-A and TOPEX-B consists in using the ERS-2 mission to remove the effect of oceanic variability previously described. In a first time, the SSH differences at crossovers between ERS-2 and TOPEX-A, and between TOPEX-B and ERS-2, are calculated over a 50-day period (from cycles 31 to 235 for TOPEX-A, and from cycles 236 to 240 for TOPEX-B) and for crossovers lower than 10 days. In a second time, the average difference of these crossovers differences is calculated, allowing us to calculate the TOPEX-A and TOPEX-B differences and assuming ERS-2 differences are negligible between TOPEX-A and TOPEX-B period. The map of SSH differences obtained (Figure 13) is significantly more homogenous than previously in Figure 12 (the same scale is used). But the detection of regional bias is not very clear. A north/south signal is likely observed on ascending passes.

Figure 13 : Map of SSH differences between TOPEX-A and TOPEX-B for ascending (on bottom) and descending (on top) passes derived from TOPEX/ERS-2 crossovers differences (TOPEX-A and TOPEX-B periods respectively cover cycles 231 to 235 and 236 to 240).

In order to better highlight a potential north/south signal, the SLA differences are plotted by band of latitudes in Figure 14. The ascending passes clearly display a regional bias depending on the latitudes from 0.5 cm in the northern hemisphere to -1.5 cm in the southern hemisphere.

Figure 14 : Map of SSH differences by band of latitudes between TOPEX-A and TOPEX-B for ascending (on bottom) and descending (on top) passes derived from TOPEX/ERS-2 crossovers differences (TOPEX-A and TOPEX-B periods respectively cover cycles 231 to 235 and 236 to 240)

## Specification of the correction

TOPEX-A/TOPEX-B SLA differences using TOPEX/ERS-2 crossovers have been plotted versus the latitudes in Figure 15. The signal is quite noisy contrary to the similar plots obtained between Jason-1 and Jason-2 or between TOPEX and Jason-1 (Figure 6). These oscillations are due to the method itself: the comparison of TOPEX-A and TOPEX-B SLA using ERS-2 SSH crossovers is not as accurate as SLA differences calculated during the Jason verification phases. In addition, these TOPEX-A/TOPEX-B SLA differences also contain ERS-2 differences between TOPEX-A and TOPEX-B phases. The hypothesis that these ERS-2 differences are negligible is not sure.

Therefore, as a result of these uncertainties, it seems not very relevant to adjust a polynomial function in order to propose an empirical regional correction homogenizing TOPEX-A and TOPEX-B MSL better, although the regional MSL bias highlighted in this study between TOPEX-A and TOPEX-B  (for ascending passes) is likely realistic.

If a reprocessing of TOPEX data is foreseen in the future, the TOPEX SSH calculation could be significantly improves and the estimation of the MSL regional bias could be checked and estimated more accurately.

Figure 15 : SLA differences between TOPEX-B and Jason-1 versus the latitudes selecting all the measurements (red curve), ascending passes (green curve) and descending passes (blue curve).

# Impact of regional bias corrections on the whole MSL time series

The impact of empirical regional bias corrections proposed in this study to better link
Jason-1/Jason-2 and TOPEX/Jason-1 MSL has been analyzed calculating the MSL trends from 1993 to 2010 and applying or not the regional bias corrections (Figure 16, Figure 17, Figure 18). The MSL trend differences range from -0.2 to +0.3 mm/yr from North to South. Considering ascending and descending passes separately, differences are higher from -0.5 to +1 mm/yr.

Notice these new corrections have no impact on the global MSL trend as displayed in Figure 19. A very slight annual signal (lower than 0.2 mm of amplitude) is observed on global MSL differences. Indeed, the application of a grid correction (constant temporally but not spatially) generates an annual signal due to the annual data coverage variation in relationship with ice sea coverage (especially in northern hemisphere).

Figure 16 : MSL trend differences from 1993 to 2010 between MSL time series without and with regional bias corrections on TOPEX/Jason-1 and Jason-1/Jason-2

Figure 17 : MSL trend differences for ascending passes from 1993 to 2010 between MSL time series without and with regional bias corrections on TOPEX/Jason-1 and Jason-1/Jason-2

Figure 18 : MSL trend differences for descending passes from 1993 to 2010 between MSL time series without and with regional bias corrections on TOPEX/Jason-1 and Jason-1/Jason-2

Figure 19 : Global MSL trend differences from 1993 to 2010 between MSL time series without and with regional bias corrections on TOPEX/Jason-1 and Jason-1/Jason-2

w

# Conclusions

The impact of empirical regional bias corrections proposed to better link
Jason-1/Jason-2 and TOPEX/Jason-1 MSL has been analyzed calculating the MSL trends from 1993 to 2010 and applying or not the regional bias corrections. The MSL trend differences range from -0.2 to +0.3 mm/yr from North to South. Considering ascending and descending passes separately, differences are higher from -0.5 to +1 mm/yr.

Notice that these new corrections have no impact on the global MSL trend as displayed. A very slight annual signal (lower than 0.2 mm of amplitude) is observed on global MSL differences. Indeed, the application of a grid correction (constant temporally but not spatially) generates an annual signal due to the annual data coverage variation in relationship with ice sea coverage (especially in northern hemisphere).

- We recommend to use the regional MSL bias correction to link together the TP, Jason-1 and Jason-2 missions.

An impact indicator clearly and easily comprehensible has been defined with 3 levels: significant impact, low impact, no impact detected. Each level is represented by a different color box.  The choice of a value indicator (significant, low or null) is quite subjective. As it depends on the application (Global MSL, regional MSL, mesoscale…), the rule to classify this impact has been defined in annex of this document (see appendix).

For the new empirical corrections developed in this study, the impact concerns only the regional mean sea level at long-term scale (trends). The impact is significant and positive for climate studies.

| Impact of new regional corrections on TP/J1/J2 times series |  |  |
| --- | --- | --- |
| Climate Applications | Temporal Scales | TOPEX+Jason-1+Jason-2 |
| Global Mean Sea Level | Long-term evolution (trend) |  |
|  | Inter annual signals (> 1 year) |  |
|  | Annual and semi-annual Signals |  |
| Regional Mean Sea Level | Long-term evolution (trend) | + |
|  | Annual and semi-annual Signals |  |
| Mesoscale | Signals < 2 months |  |

Definition of the indicator value

In this table, the choice of the indicator value is defined for each climate applications and temporal scales. The thresholds defined here are valid for time series long enough (> 7 years). If time series is too short, the thresholds have to be majored.

| Climate Applications | Temporal Scales | Definition of the indicator value |  |  |
| --- | --- | --- | --- | --- |
|  |  | Significant impact | Low impact | No impact detected |
| Global Mean Sea Level | Long-term evolution (trend) | Trend >0.15 mm/yr | Trend> 0.05 mm/yr | Trend< 0.05 mm/yr |
|  | Inter annual signals (> 1 year) | Amplitude> 0.5 mm | Amplitude> 0.2 mm | Amplitude< 0.2 mm |
|  | Annual and semi-annual Signals | Amplitude> 1 mm | Amplitude> 0.2 mm | Amplitude< 0.2 mm |
| Regional Mean Sea Level | Long-term evolution (trend) | Trend > 0.5 mm/yr | Trend> 0.1 mm/yr | Trend< 0.1 mm/yr |
|  | Annual and semi-annual Signals | Amplitude> 5 mm | Amplitude> 0.5 mm | Amplitude< 0.5 mm |
| Mesoscale | Signals < 2 months | Crossovers Variance differences > 1 cm² | Crossovers Variance differences > 0.2 cm² | Crossovers Variance differences < 0.2 cm² |
| Specific regional areas of main interest for climate studies: |  |  |  |  |
| Coastal areas | Long-term evolution (trend) | Trend > 0.5 mm/yr | Trend> 0.1 mm/yr | Trend< 0.1 mm/yr |
|  | Signals < 2 months | Crossovers Variance differences > 1 cm² | Crossovers Variance differences > 0.2 cm² | Crossovers Variance differences < 0.2 cm² |
| High latitudes | Long-term evolution (trend) | Trend > 0.5 mm/yr | Trend> 0.1 mm/yr | Trend< 0.1 mm/yr |
|  | Signals < 2 months | Crossovers Variance differences > 1 cm² | Crossovers Variance differences > 0.2 cm² | Crossovers Variance differences < 0.2 cm² |

Definition of altimetry standards

The definition of altimetric standards applied in MSL calculation to perform this study is defined in the table below.